Collector – Android

- Cancel—Discards the feature if you are collecting a new feature, or discards your edits if you are editing an existing feature.

- Collect Settings—View and update the required accuracy and streaming interval used during collection.

- View Map—Shows the map if you are viewing the attributes list.

- Add Attachment—Adds attachments such as photos and videos to a feature if attachments are enabled in the map.

- Submit or Update—Saves the feature and the data you collected about it. Submit (shown here), appears when you are creating a new feature. Update (not shown) appears when you are editing an existing feature.

- Feature type—Indicates the type of feature you are collecting.

- Fields—Enter information about the feature in each of the fields shown.

- Current Accuracy—Like the Location Accuracy badge, indicates current horizontal accuracy as well as whether the current accuracy is below the threshold required to use the GPS to collect data. Unlike the Location Accuracy badge, selecting this does not display more details about your current GPS location.

- Collect Attributes—Shows the Attributes list.

- Use My Location—Updates the feature's location or adds a new vertex using the GPS.

- Delete Vertex—Deletes the currently active vertex of your polygon or line feature. It only displays when you are collecting or editing a polygon or line feature.

- Undo—Reverts the last change you made to the shape of a feature. For example, if you update the location of a feature or add a point to a shape you're drawing, selecting Undo discards these changes. However, if you edit a field in the Attributes list and select Undo, your field edits are unaffected and the last change made to the feature's shape is undone.

- Delete—Discards the line or polygon feature you are collecting or editing. This tool only displays when you are working with polygon or line features.

- Stream—Automatically collects a shape using your device's GPS, allowing you to walk or drive along a feature and capture its shape without manually placing points. When selected, it changes to Pause and you can't manually edit the shape. To make manual edits, pause streaming, make your updates, and restart streaming. You can only stream when collecting or editing a polygon or line.

- Map—Allows you to create and edit the feature's shape.